Drinking water system Report

| TO: | CAO, Council & Public Works Manager |
| --- | --- |
| FROM: | Brad Faught, Quality Management System Representative |
| DATE: | October 7, 2025 |
| SUBJECT: | 3rd Quarterly Report 2025 |

Background

Laurentian Valley Water Works manages just over 14 kilometres of water main and branch lines in the Township. Laurentian Valley Water Works received its Certificate #2893 as a Water Distribution System Class 1 in November of 2000. On May 31, 2011 Drinking Water Works Permit 192-201 and License Number 192-201 was issued by the Ministry of Environment for the Laurentian Valley Distribution System. On February 8, 2021 the License & Permit were renewed. The Drinking Water System Information number for Laurentian Valley Water Works is W260007465. This is the third report for 2025.

Laurentian Valley Township has a water servicing agreement with the City of Pembroke. The water is purchased from the City and distributed in the Stafford Village area, part of Pembroke Street East and part of Pembroke Street West. The City treats and tests the water at the Pembroke Filtration Plant on Riverside Drive and tests the water in their distribution system. The Municipality tests the water in its distribution system.

This report and the Certificates of Analysis are available to the general public at the front counter of the Municipal Office at 460 Witt Road during office hours. Information on the City Water Works can be accessed at www.pembroke.ca.

Strategic Directions Alignment:

Core Values

|  | Value for money |  | Professionalism |
| --- | --- | --- | --- |
|  | Act with integrity | X | Transparency |

Strategic Directions

|  | Embrace Rural Identity |  | Invest responsibly in assets & programs |
| --- | --- | --- | --- |
|  | Prioritize Communication |  | Enhance active & social living |

Mission/Vision/Other

Testing

The weekly water samples were sent to Caduceon Laboratories Inc. in Ottawa for analysis. Caduceon is an ODWS Accredited laboratory. The results of the analysis are emailed to the Municipality. The following chart is a summary of those test results.

Analysis for the period of July 02, 2025 – October 1, 2025

| Parameter Tested | MAC, IMAC or Standard | # of Samples | # of Adverse Results | Normal Range | Note # |
| --- | --- | --- | --- | --- | --- |
| E, Coli | Nil | 30 | 0 | Absent |  |
| Total Coliforms | Nil | 30 | 0 | < 1/100ml |  |
| Heterotrophic Plate Count | 500 CFU/ ml | 10 | 0 | <2 CFU/ml – 4 CFU/ml |  |
| Residual Chlorine | .05 mg/ml | 104 | 0 | 0.07 – 1.00 mg/l |  |
| Trihalomethanes | 0.10 mg/liter | 1 | 0 | 0.111 mg/liter | 4/4 avg |
| Haloacetic Acid | 0.08 | 1 | 0 | 0.0595 | 4/4 avg |

In the third quarter of 2025, the municipality paid Caduceon $2324.72 for laboratory services (HST included).

Complaints

There were no complaints in the third quarter.

Staffing & Training

Laurentian Valley Waterworks has 1 Water Quality Analyst Representative, 3 operators certified for Water Distribution 1 and one Operator-in-Training.  Three operators attended the Maintenancefest in Kingston, Ontario which was a hands-on training for practical use for in-field operations.

Maintenance

There were 55 locate requests completed in the third quarter, of which 26 identified water lines to be marked.

Two malfunctioning water meters were changed out in the Karen Street and Pembroke Street West water chambers

A valve box was repaired on Glen Street

A curb box and rod was replaced in a driveway on Glen Street

Ministry of Environment, Conservation & Parks

There was an exceedance on the Trihalomethane (THM) readings for Home Depot in the third quarter.  Trihalomethane’s are a chemical compound that can be formed when water is disinfected with chlorine.  THM’s occur when chlorine reacts with organic matter in water.  Extensive flushing was completed and numerous tests were completed.  Unfortunately, our water is supplied by the City of Pembroke and we have no means of rectifying this issue other than to complete hydrant flushing on that line.  It was reported to the MECP and also the Renfrew County and District Health Unit.  Testing will be conducted again on November 17, 2025 to ensure levels have gone down as per the direction of the RCDHU.

Construction

Nothing to report.

Drinking Water Quality Management System

N/A

Summary

In the third quarter of 2025, the water works purchased 64,028.72 cubic meters of water (2024 was 57,876.30) from the City of Pembroke distributed to homes and businesses.

Respectfully Submitted

Township of Laurentian Valley

“Original Signed”

Brad Faught

Quality Management System Representative

Acronyms

ODWS – Ontario Drinking Water Standards

MAC – Maximum Acceptable Concentration

IMAC – Interim Maximum Acceptable Concentration

MECP- Ministry of Energy, Conservation and Parks

RCDHU – Renfrew County and District Health Unit